Masterarbeit LRS-MA 25/

Aufgabenstellung für

cand. -M.Sc. Joel Agustin Sanchez

Uncertainty-Aware MPC: Probabilistic SDFs and Active Visibility for Dynamic Clutter

1. 🎯 Objective and Context

The objective of this Master's Thesis is to develop and validate an advanced Model Predictive Control (MPC) based motion planner. This planner must enable a redundant robotic agent (R1) to perform motion planning in dynamic and cluttered environments where the environment model is highly uncertain and subject to real-time changes. The key innovation lies in integrating a probabilistic environment model (Probabilistic SDF) with an MPC framework that actively optimizes for target visibility.

The devised planner is to be experimentally validated on the available 7-DoF Franka Panda manipulators in the LRS robotics lab using the specified "High-Low Table" scenario, which features dynamic interaction with a second manipulator (R2).

### 2. 📚 Task: Literature & Baseline Analysis

- Review of SDF/TSDF Planning: Survey state-of-the-art use of ESDF/TSDF fields for real-time collision avoidance in optimization-based motion planning.

- Review of Probabilistic Mapping: Investigate Probabilistic Voxel-Maps with Bayesian Updates and Memory Decay mechanisms for dynamic environment representation.

- Survey of Active Perception: Analyze gradient-based Active Visibility techniques, specifically Differentiable Ray-Casting, suitable for MPC integration.

- Baseline Analysis: Analyze the vision cost and collision model limitations of the existing C2P-MPC baseline (Convex Hulls, Passive Vision).

### 3. 💻 Task: Technical Design & Implementation

#### A. Environment Modeling & Safety Layer

- Probabilistic Voxel-Map & ESDF: Implement a Probabilistic Voxel-Map with Bayesian Updates and Memory Decay. Convert this map to a real-time ESDF field for continuous, differentiable MPC collision gradients.

- Dynamic Safety Model: Integrate the Neural Proxy collision model to ensure precise and efficient collision avoidance between R1 and the dynamic disturber R2.

#### B. Active MPC Core

- ESDF-based Collision Cost: Reformulate the MPC cost function to use the ESDF gradient for collision avoidance, replacing static convex hull constraints.

- Differentiable Active Exploration: Implement the Differentiable Ray-Casting cost term within the MPC to actively optimize the Eye-in-Hand camera's Line-of-Sight (LoS), forcing emergent "peeking" motions around occlusions.

- Framework Integration: Ensure the new cost terms are efficiently integrated and compatible with the B-spline trajectory transcription and the CasADi/IPOPT solver.

### 4. 🧪 Task: Validation and Analysis

- Use Case Validation (Simulation/Hardware): Design and execute experiments in the "High-Low Table" scenario to demonstrate:

  - Ghost Obstacle Clearance: Validation of Memory Decay by clearing areas recently vacated by R2.

  - Proactive Peeking: Demonstration of the Active Visibility Cost generating emergent exploration motions to resolve occlusions.

  - Dynamic R2 Safety: Validation of the Neural Proxy for safe, precise R1-R2 avoidance.

- Sensitivity Analysis: Determine the effect of key parameters: Voxel-Map Decay Rate, ESDF generation frequency, and the weighting of the Active Visibility Cost on planning performance.

### 5. 📝 Task: Documentation & Outlook

- Summarize findings, analyze the strengths and limitations of the developed planner, and provide a comparison to the C2P-MPC baseline.

- Discuss future improvements and potential extensions.

- Documentation and presentation of the thesis work.

Start: 2025

The thesis work is conducted at the Institute of Control Systems, University of Kaiserslau-tern-Landau and supervised by me and my research associate M.Sc. Chen Cai.

Prof. Dr.-Ing. S. Liu